Терехов Максим, ІК-93, ФІОТ
Лабораторна робота №1
НАЛАШТУВАННЯ МЕРЕЖНОГО ОТОЧЕННЯ ТА ТЕСТУВАННЯ ПРОТОКОЛУ ARP

Варіант завдання – 0 (Переконатися в отриманні МАС-адреси по відомій IP-адресі.)

Теоретичні відомості:

- Встановлення утиліти для сніффінгу трафіка Wireshark

- Запуск утиліти

- Тестовий пінг

- Зібранний трафік

- Перевірка таблиці apr та видалення записів з неї

- Пінг ip-адреси всередині мережі

- Зібрані пакети з запитом mac-адреси

- Відповідний запис в apr-таблиці

TEST-CASE

Topic – Wireshark functionality. Virtual machine gateway IP and MAC address resolving

Description - Make sure to get a vm machine gateway IP and MAC address

Attachments – ARP_Tests.pcapng

Version:

net-tools 2.10-alpha

wireshark 3.6.2

Setup:

- Virtual machine with Ubuntu OS

- PC1-adapter-gateway-internet

PC1 IP – 192.168.0.1

Preparation:

| № | Description | Action | ER |
| --- | --- | --- | --- |
| 1 | Check net-tools and wireshark installed | dpkg -s wireshark dpkg -s net-tools | installed - ok |
| 2 | Clean arp table | sudo arp -d | arp – a doesn’t print anything |
| 3 | Launch wireshark | Sudo wireshark | Wireshark window is opened |

Steps:

| № | Description | Action | ER |
| --- | --- | --- | --- |
| 1 | Pick Wireshark interface (any or enp0s3) |  | Capture menu is opened |
| 2 | Open additional terminal |  |  |
| 3 | Ping some external addresses from new terminal | ping google.com | 64 bytes from <host> (<ip-adress>): icmp_seq=2 ttl=118 time=16.6 ms Will be printed repetatly |
| 4 | Check new packets captured | Open wireshark window | New rows will appear in capturing table |
| 4 | Stop packets capturing | Click “Stop packets capturing” button | New rows will stop appearing |
| 5 | Filter ARP protocol | Write arp in filter input field | There have to be 2 packets with ARP protocol |
| 6 | Check packets info | Search “info” column | First packet info has been “Who has <IP>” and second info is “<IP> is at <MAC>” |
| 7 | Close or minimize the wireshark window | You can save capturing file if you want to work with it lately |  |
| 8 | Check arp table | arp -a in terminal | _gateway <IP> at <MAC>[ether] on enp0s3 |
| 9 | Compare results |  | IP and MAC from Wireshark and ARP must be identical |

Висновки:

В ході лабораторної роботи ми написали тест-кейс для тестування роботи ARP протоколу та вивчили основні принципи його роботи, та принцип сніффінгу трафіку за допомогою утиліти Wireshark